Comparison of Duke Energy and Carolina Power & Light’s CBM/TRM Methodologies

|  |  | Duke Energy | Carolina Power & Light |
| --- | --- | --- | --- |
| CBM | Definition | Same as NERC | Same as NERC |
|  | Value (Imports Only) | Zero on all interfaces | Zero on all interfaces |
|  | Reason | - No significant transfer limits - TTC calculations use reduced line ratings (12 hr vs. 1 hr when operating the system) - TTC calculations are based on single worst transmission contingency and adverse generation participation | - Loss-of Load Expectation (LOLE) of 1 day in 10 years requires an emergency transfer capability between 1500 MW and 1800 MW - Required generation reserve is accounted for in TRM |
| TRM | Definition | Same as NERC | Same as NERC |
|  | Value (Imports) | - Opposing control area’s share of the VACAR reserve sharing requirement | - Opposing control area’s share of the VACAR reserve sharing requirement [TRM (RS)] plus TRM-Additional Parallel Path Flow Impact [TRM (APPFI)] |
|  | Value (Exports) | - Reserve obligation to meet the VACAR Reserve Sharing Agreement | - Reserve obligation to meet the VACAR Reserve Sharing Agreement |
|  | Reason | - To declare TRM there should be a contractual obligation for reserves, such as the VACAR Reserve Sharing Agreement | - Account for variations in generator dispatch, parallel path flows, and operating reserves, such as the VACAR Reserve Sharing Agreement |
|  | Selling | - Not sold on a firm or non-firm basis, as doing so would degrade system security | - Not sold on a firm or non-firm basis, as doing so would degrade system security |
| CBM/TRM | Availability to LSEs | - Not available for use on a firm basis by market entities, including Duke Energy’s affiliated marketer - In response to a generation emergency, the System Operating Center uses TRM for VACAR reserve sharing purposes to benefit all control area LSEs | TRM available for: - Loss of firm resource invoking an Emergency Reserve Sharing Agreement - Declaration of an Energy Emergency Alert (EEA) due to insufficient resources - LSE has exhausted all other options and can no longer provide its customers’ expected load requirements. |

2005 VACAR Reserve Sharing Obligations

| 2005 Reserves | Adjusted Peak Load (MW) | Largest Resource (MW) | Contingency Reserve % | Contingency Reserve Commitment (MW) |
| --- | --- | --- | --- | --- |
| Duke | 17926 | 1135 | 0.2971 | 506 |
| CP&L | 11699 | 900 | 0.2129 | 363 |
| SCPSA | 4781 | 590 | 0.1136 | 193 |
| SCE&G | 4574 | 636 | 0.1172 | 200 |
| DVP | 16507 | 925 | 0.2592 | 441 |
| Total | 55487 | 4186 | 1.0000 | 1703 |

Current Duke Energy and Carolina Power & Light CBM/TRM Values

| Control Area | From | To | CBM | TRM or TRM (RS) | TRM (APPFI) |
| --- | --- | --- | --- | --- | --- |
| Duke Imports | PJM | Duke | 0 | 0 | N/A |
|  | CP&L East | Duke | 0 | 363 | N/A |
|  | CP&L West | Duke | 0 | 0 | N/A |
|  | SCE&G | Duke | 0 | 200 | N/A |
|  | SCPSA | Duke | 0 | 193 | N/A |
|  | TVA | Duke | 0 | 0 | N/A |
|  | Southern | Duke | 0 | 0 | N/A |
|  | SEPA | Duke | 0 | 0 | N/A |
|  | Yadkin | Duke | 0 | 0 | N/A |
| Duke Exports | Duke | PJM | - | 0 | N/A |
|  | Duke | CP&L East | - | 306 | N/A |
|  | Duke | CP&L West | - | 200 | N/A |
|  | Duke | SCE&G | - | 506 | N/A |
|  | Duke | SCPSA | - | 506 | N/A |
|  | Duke | TVA | - | 0 | N/A |
|  | Duke | Southern | - | 0 | N/A |
|  | Duke | SEPA | - | 0 | N/A |
|  | Duke | Yadkin | - | 0 | N/A |
| CP&L East Imports | PJM | CP&LE | 0 | 441 | 487 |
|  | Duke | CP&LE | 0 | 506 | 0 |
|  | SCE&G | CP&LE | 0 | 200 | 0 |
|  | SCPSA | CP&LE | 0 | 193 | 0 |
| CP&L East Exports | CP&L East | PJM | - | 363 | 0 |
|  | CP&L East | Duke | - | 363 | 0 |
|  | CP&L East | SCE&G | - | 363 | 0 |
|  | CP&L East | SCPSA | - | 363 | 0 |
| CP&L West Imports | PJM | CP&L West | 0 | 0 | 42 |
|  | Duke | CP&L West | 0 | 0 | 200 |
|  | TVA | CP&L West | 0 | 0 | 0 |
| CP&L West Exports | CP&L West | PJM | - | 0 | 0 |
|  | CP&L West | Duke | - | 0 | 0 |
|  | CP&L West | TVA | - | 0 | 0 |